ZEBRA BATTERY

Winflash version 1.16r2 Installation notes

Version :	1.16r2

Date :		10.12.2010

Author :	AB

Winflash has to be installed on a PC after installation of HW interface.

Included in the folder CDROM:\DiagnosticKitxx\WinFlash\Ver_116r2 there are:

1_winflash_116r2.exe, the installation package for STW Winflash116r2, and

2_STW_DLL_WF.zip with STW DLL interfaces and INI files to handle HW interface: PeakCAN-USB, Vector CANcardXL, CANcardX, CANpari.

3_WinflashTroubleshooting.txt, with some suggestion about problems that can be occurred.

In order to install Winflash, please follow the steps below:

- Verify that in “Control Panel” the applet “CAN Hardware” has “Active Device” set to “USB”, as showed below. Otherwise, please set USB as Active Device and press Apply button.

- Execute 1_winflash_116r2.exe.

  - Select Remove if another previous version is already installed and then run again 1_winflash_116r2.exe.

  - Press Next button twice.

  - Press Yes to confirm the license.

  - Press Next to confirm as installation path C:\Programmi\STW\KEFEX Winflash.

  - Select Compact installation and then press Next button.

  - Select Configure Manually option and then press Next.

  - Press OK to complete the installation.

- Decompres 2_STW_DLL_WF.zip  into C:\Programmi\STW\KEFEX Winflash\Winflash.

  - Press Yes to All in order to replace all existing files.

  - Now in C:\Programmi\STW\KEFEX Winflash\ Winflash there are all the files used by Winflash.

- After the installation run Winflash from Start  Programmi  STW  KEFEX WinFlash   Winflash

- And check the following setting:

For VECTOR interface

Comm-DLL: C:\Programmi\STW\KEFEX Winflash\Winflash\stwvec32.dll

Configure: select the used Vector channel.

For PEAK interface

Comm-DLL: C:\Programmi\STW\KEFEX Winflash\Winflash\stwpeak2.dll

Configure: select TestNet as Setup network with 125K bit rate and 11bit ID to receive at 0x000/0x7FF

Local ID = don’t care

Company ID = MD

Send ID= 81

Bitrate= 125K usually

Type of HexFile= C application (all sectors)

SendResetRQ= disabled

Timing Delay= 500 micro sec

Start Time= 3s usually or higher

Node’s SN = don’t  care

Only if you do modifications, save the current configuration as FZSONICK with command  File  Save Current Configuration.

To download software on one BMI:

- select Search Node,

- turn on the ignition key when required

- in Windows Results flag the BMI where download

- select Flash node to start the download.

- (Ignore permanently hex/device ID consistency)

At the end of the download:

- Close winflash

- Turn ignition key at off for at least 20sec and then turn on ignition key and leave at on for at least 20sec.

IMPORTANT NOTE:

If an error occurs after search nodes command while PEAK CAN USB hardware is used, please see the file “WinflashTroubleshooting.txt” in the folder “C:\Programmi\STW\KEFEX Winflash\Winflash”.